Màster en Métodes Numèrics 		 			       Aitor Bazán Escoda

Universitat Politènica de Catalunya							    10/01/19

Assignment 1: GiD Homework

# Process followed

- SPD file implementation.

SPD file is created in order to prompt the necessary parameters in GiD’s interface through a ‘tree view’, and materials, conditions, units are associated to groups of entities. In fact, this is the main configuration of the data tree (XML based)

  - Materials file with the material densities is also generated.

  - Images folder with materials, units and constraints icons to be prompted in the program’s interface is set up.

- TCL file implementation.

TCL file is generated so as to write the calculation file. By this mean, the solver gets the input data file and executes the calculations.

- Program execution from the same GiD’s interface.

- Post processing.

# Writing procedure

- SPD file.

It needs all the data necessary to perform an analysis. This is: boundary conditions, loads, materials, loadcases, etc. As it is shown in the same file, the elements form a tree-structure that starts at the root and branches to the leaves1.

First it is defined the main unit field. For the problem type Frame3DD units have to be referenced to the International System (SI). Moreover, if the examples run require any specific unit to be automatically promped by the interface, this field is used to do so. However, this field could be left empty if default units are enough.

Angle is given in radians as default unit. Therefore, as it has to be shown in degrees, this will be the active unit in the spd file. The same would happen to Temperature units. They are given in Kelvin but need to be translated to degrees Celsius.

In the case, the units are already in the SI, but are derived from the basic units, those from the International System, they are needed to be referenced as well. In case millimetres is required instead of meters, the conversion would be done the same way as the others.

This is done by changing the unit_magnitude field of Angle, n=”Angle” (where n is the name used to reference the field), pn=”Angle” (pn is the label visualized by the user), default=”rad” (this is the default unit), SI_base=”m/s^2” (base unit), active=”º” (unit wanted to be visualized). Then, the field unit, renaming the unit to be used is written. Factor p will give the priority for the visualization of the unit. If the priority is:

- P = 1, it gives maximum priority.

- P = 2, is given always visualization in the combobox to facilitate the selection of the most used units.

- P = 3, lowest priority. The visualization of the units with such priority is given when clicking on ‘More units’ option.

Typically p = 1 will be used.

<unit_magnitude n="Angle" pn="Angle" default="rad" SI_base="rad" active="°">

<unit n="°" pn="degree" p="1" units_system="SI"/>

</unit_magnitude>

Once the automatic visualization of the required units is done, it is time to define the basic units to be asked by the program when referencing to physical quantity.

This is to say: units of length will be given in L, force in F, Acceleration in Acceleration. This is something defined in GiD customization manual.

When units are already defined, the implementation of the conditions to be promped in the data tree is required.

To do so, it will be defined first the condition for reactions. This condition is applied over the entity type point, which applies to entity node. The value in n field will be required then for the TCL file to prepare the input file. Inside condition field, it is to be specified the values asked for this condition to the user. In this case, possibilities for the user are 1 or 0 in function of the node if it is fixed or it is not.

Editable option set to 0 allows the user to change it to 1. Also, to be clarified, icon parameter set to “constraints” allows to visualize the icon for such condition.

Then it comes Frame Element condition which is applied over the entity type line, which also applies to entity element. Inside the condition we set the value field for the required inputs. The v parameter should be left to 0.

Once these two conditions are set, it starts the part where the load cases are defined in.

Static loads is the container field in which the blockdata for the load cases will be implemented.

In the definition of the Blockdata, there are three parameters to be commented here:

- Sequence: set to 1 it allows the user to duplicate or copy the field in order to create several load cases with its value and condition included. As explained in the customization manual, if this parameter is activated, it will be possible for the user to create consecutive repetitions of the full block data so as to represent loadcases with all the conditions inside.

- Sequence_type:

  - Any: the list can be void.

  - Non_void_disabled: At least there needs to be one element. It can be disabled.

  - Non_void_deactivated: At least there needs to be one element. It can be deactivated.

- Editable_name: can be void or unique. ‘Unique’ means that it is not possible to use the same name (pn field) for two different ‘sequence’ ‘blockdata’.

Once highlighted these points, for the present case, are used sequence set to 1, sequence_type to any and editable_name to unique.

Next there are different loadcases defined. The cases where the condition needs to be applied on points/nodes are:

- Nodal weight.

- Prescribed displacements.

On the other hand, the cases where the condition is applied over line/elements are:

- Uniform weight.

- Thermal load.

- Trapezoidal load.

- Self weight.

- Internal load.

Here the blockdata ends.

Then, it begins the definition of the container for extra options such as:

- Shear deformation: takes 1 and 0 values in function of weather it has to be activated or not accordingly.

- Geometric stiffness. Same options as shear deformation.

- Dynamic modes.

- Animated modes. It has to be less or equal than dynamic modes and, of course, never less than 0.

- Mmethod. This is subspace. 1 for Jacobi, 2 for Stodola.

- Lump. As the mass matrix type. 0 for consistent mass matrix, 1 for lumped mass matrix.

- Pan. The pan rate. 0 for no panning.

- Tol. The mode shape tolerance.

- Shift. For unrestrained structures.

- Exagg_modal. Modal mesh deformations.

- Exagg_static. Exaggeration factor for static mesh deformations.

- Scale. Zoom scale for 3D plotting.

- Dx. This is the x-axis increment for frame element internal force data.

Then, in the same container of extra options, the condition for extra node inertia and extra element inertia are set.

It is to say, as well, there is an implementation taken from cmas2d_customlib in which Shells function is implemented.

- TCL file

All the used commands are commented on the same script. Refer to them for more information.

For the definition of the procedure WriteCalculationFile, the input data format is followed.

First the definition of the coordinates in 3D with the radius, in case of spherical geometry. Mesh units are also prompted out.

Then, for reactions, their fixed or unfixed nodes are written.

Frame elements are written in terms of elements since what is asked through the spd file is to apply such condition on lines/elements.

Following the pattern of the Pyramid case in Frame3DD website, following extra options are shown:

- Shear; Geom; exagg_static; scale; dx;

Some others will be defined at the end of the procedure.

What it is done now is in order to compute the number of loadcases.

If in the command palette in GiD, the following commands are executed, the results on the prompt are:

- -np-  set document [$::gid_groups_conds::doc documentElement]

- -np-  set xpath "/frame3dd_data/container\[@n = 'staticLoads' \]/blockdata"

- -np-  set xml_nodes [$document selectNodes $xpath]

- -np- W $xml_nodes   //to visualize the data for the nodes.

What prompted is:

domNode000002B1FDEE80F0 domNode000002B1FDEE1C10 domNode000002B1FDEC9050

- set case_numb [llength $xml_nodes] //this way the number of nodes printed before is obtained. Llength calculates the length of the variable xml_nodes.

- Variable m is set to 1. It will be the incremental counter of load cases.

Then, for each loadcase the conditions asked through the GiD’s interface are printed in the input data file.

For the case of self-weight it is prompted for the whole structure (globally). When applying it into the structure, it will just be applied to one frame element.

After self weight, there is nodal weight.

As a resume, since the computing structure is nearly the same for all the conditions, it needs to be highlighted the following steps, in terms of pseudocode:

For each load case:

- Set path to nodal weight groups.

- Set variable groups to the number of nodes/elements from the path got above for the current load case $m.

- Set variable number of elements to 0.

- Set variable format of dict to dict create.

- Then, for each group of nodal weights:

  - Get the name of the group.

  - Select the nodes from the current group.

  - Get the value of the nodes selected in the previous step.

  - Do this for each values asked through spd file.

  - Set the format of the values needed to prompt.

  - Set the variable created “format of dict” so as to merge the number of current node with its properties.

- Set number of elements so to count the number of nodes used.

- If the number of nodes is greater than 0, prompt number of nodes and the variable “format of dict”

- Else, prompt number of nodes. This will be 0.

Increment variable m at the end of load case.

(*)Indeed, for the calculation of internal concentrated loads, if dx, this is the x-axis increment for frame element internal force data, is -1, it will not be computed.

Then the document asks for some more extra options:

- Dynamic_modes.

If Dynamic is 0, all the remaining extra options will not be necessary.

Else:

- Mmethod.

- Lump.

- Tol.

- Shift.

- Exagg_modal.

- Here it is also asked for the condition: nodes and elements with extra inertia.

After that, condition: number of modes to animate is proposed.

If the number of modes to animate is greater than 0:

Treat the variable number of nodes as a list and append each of the value arguments to that list as a separate element. In this case, the variable a is a unit counter. Therefore, setting the counter inside a for loop with the lappend command, the function will prompt a list from 0 to number of nodes.

Else:

Prompt 0.

- Finally, the program also asks for the pan rate.

End of the input data file.

# Found and solved problems

Radius

In case of dealing with 3D coordinates, if radius (for sphere) is not the last value prompted in coordinates format, everytime the user inputs negative coordinates, the solver crashes.

Reactions

Not using the appropriate xml function in the spd file led to errors when writing the input file. Rethinking the format and checking the customization help was the solution.

Number of modes to animate

The incapability of finding the correct loop to gather the values into vector led to errors.

Finding the correct algorithm with the help of the tcl/tk manual.

Compiling errors

When executing from GiD’s interface (F5), there was showing a problem with no previous understandings.

Copying the .dat file into the exe file in the problem type folder and executing frame3dd from the command palette was the solution. Then, a change given by the professor solved the issue.

# Examples used

Truss 2D Pratt Truss case was executed.

As the input data file is written in Imperial System units, before loading conditions, the units are changed.

The post process is done in gnuplot and in GiD. The file is attached as v.gid case.

Also, Pyramid-Shaped Frame is used to test the problemtype.