GiD Problemtype-Frame3DD

Yuyang Wang

Master on Numerical Methods in Engineering

Universitat Politècnica de Catalunya,BarcelonaTech

1. Introduction

Frame3DD is free open-source software for static and dynamic structural analysis of 2D and 3D frames and trusses with elastic and geometric stiffness. Gid is a universal, adaptive and user-friendly pre and postprocessor for numerical simulations in science and engineering.

The goal of this report is to issue a way to run Frame3DD.exe as a solver in GiD. In this case, GiD works for output essential data which is required by Frame3DD. Frame3DD process this data and output the result.

2. File Creation

2.1 Creating the General Materials File

This file stores the physical properties of the material under study for the problem type. According the user manual, here we have nine kinds of materials with such properties: Young’s modulus, Shear modulus, thermal expansion coefficient, mass density, and modulus per density. The file “Frame3DD.mat” uses the following format:

MATERIAL: Steel_A36

QUESTION:Young's_Modulus

VALUE:200000

QUESTION:Shear_Modulus

VALUE: 79300

QUESTION:Thermal_Expansion_Coefficient

VALUE:11.7e-6

QUESTION:Mass_Density

VALUE:7.85e-9

QUESTION:Modulus_Per_Density

VALUE:2.55e13

END MATERIAL

The green font is the name of the material, and the blue font is the specific property and its value. Following this format, we create the file “Frame3DD.mat”.

2.2 Creating the General File

The file “Frame3DD.prb” contains genral information for the calculating module, such as the units system for the problem, or the type of resolution algorithm chosen. This file uses the following format:

PROBLEM DATA

QUESTION: Name of the parameter.

VALUE: The default value of the parameter.

END PROBLEM DATA

2.3 Creating the Condition File

Create the “Frame3DD.cnd” file, which specifies the boundary and/or load conditions of the problem type in question. In this case, we have the following loads according to the user manual: gravity loads, joint loads, uniformly-distributed loads, frame element point loads, trapezoidally-distributed loads, thermal loads and prescribed displacements. Besides, according to template file, some points are restrained, so this condition is also included. Enter the boundary conditions using the following format:

NUMBER: Identifier of the condition

CONDITION: Name of the condition

CONDTYPE: Type of entity to which the condition is to be applied, such as “over points”, ”over lines”, ”over surfaces”, and ”over volumes”.

CONDMESHTYPE: Type of entity of the mesh to which the condition is to be applied. The possible parameters are “over nodes” or “over elems”.

QUESTION: Name of the parameter of the condition.

VALUE: Default value of the parameter

END CONDITION

In fact, the condition file always works for assigning relevant parameters needed in the template file.

A full condition file is written in Frame3DD.cnd.

2.4 Creating the Template File

This file will define format of the .dat text file created by GiD. It will store the geometric and physical data of the problem. The .dat file will be input to the calculating module.

To fulfill this file, we primarily refer to temple file given in the user manual.

A full template file is written in Frame3DD.bas.

2.5 Creating the Execution File of the Calculating Module

This file connects the data file to the calculating module(the Frame3DD.exe program). When the GiD Calculate option is selected, it executes the .bat file for the problem type selected. In this case, the Frame3DD.bat file is,

del %2\%1.out

del %2\%1.post.res

del %2\%1.err

del %2\%1.log

rem ErrorFile: %2\%1.err

rem OutputFile: %2\%1.log

%3\frame3dd.exe -i %1.dat -o %1.out -x > %1.log 2> %1.err

3. Example

In order to illustrate the problem type, the following examples are shown.

3.1 exB from the Frame3DD website

According to the input data file for this example, we create the geometry and assign relevant conditions through GiD, and after calculation. A file named exB.gid was obtained to show all the results. See further results; please open the file with GiD.

The video modal rotation.avi and deformation were displayed as dynamic analysis!

To show the result from GiD, three deformation pictures with color bar were exported here. we can verify from the condition that the results is true.

For node 1,node 2, node 4 and node 5, they are constrained and cannot move, so the displacement is always zero.

- In step 1, the entity is under the gravity force on its body and a punctual force on node 3, so the deformation direction can be get by comparing its original state(figure1).

- In step 2, the entity is under the gravity force on its body, uniform loads on element 1 and 4, trapezoidally distributed loads on element 2 and 3, temperature loads on elmemnt 1. So the deformation results is shown in the following picture.

- In step 3, the similar analysis is as in step 1 and step 2, and the deformation results is also shown in the following picuture(figure 2).

- Besides, in the attachment, there are some videos generate from GiD about the animation process!

- The document file “exB.gid” has all the analysis information for readers to verify!

Figure 1 Deformation results for step 1

Figure2 Deformation results for step 2 and step 3

3.2 Personal Created Example

We try to analyze the following model, one simplified house(figure3), under its self-weight and some uniform fore.

Figure3 A house to be modeled

http://shed4plans.co.vu/shedfree/wp-content/upload/2013/12/shed/shed-building-plans-8x12-diy.jpg

Firstly, the model should be sketched in GiD.

In real case, the house is under body force and wind force. For this mode, we assign gravity force, and a uniform force in X direction to see its deformation under such deformation.

The deformation results are obtain by using the problem type Frame3DD (Figure4).

To get more results about this model, such as reaction force, deformation in a specific direction, please open the file named House.gid

Figure 4 Deformation results from GiD problem type Frame3dd

4 Conclusion

A GiD problem type has been finished, which can be used to analyze different types of frames, statically or dynamicly.

During this process, I learned a lot about GiD. Thanks to the teacher Enrique Escolano, he gave me much guidance which is very helpful to advance my level.

5 Reference

[1] Ribó, R., Pasenau, M., Escolano, E., Ronda, J. S. P., Gonzalez, L., & Rosa, E. (1999). GiD user manual. CIMNE, Barcelona.

[2] Ribó, R., Pasenau, M. A. R., Escolano, E., Ronda, J. S. P., & González, L. F. (1998). GiD reference manual. CIMNE, Barcelona.